# Procedure Profile

Department – Product Information    Pertaining To – Tile Ratings

Last Update – 5/04/2007    Last Revised BY – Kurt Duitsman

Hardness Rating – Mohns Relative Hardness Scale

The Mohs scale of mineral hardness characterizes the scratch resistance of various minerals through the ability of a harder material to scratch a softer material. The higher the Mohs scale number the harder the surface or bisque.

1. Talc                                 6. Orthoclase Feldspar, Glass, Steel

2. Gypsum                               7. Quartz, Garnet, Hardened Steel

3. Calcite, Gold, Silver, Penny         8. Topaz

4. Fluorite, Platinum                   9. Corundum – Aluminum Oxide Mineral

5. Apatite, Iron                       10. Diamond

PEI (Porcelain Enamel Institute) Scale – Wear Rating

The PEI rating is a vital tool when deciding which tile is best for you because it classifies the ceramic surface in terms of how much wear the tile can bear without damaging its appearance.

CLASS 0 - Tiles technically unsuitable for floors.
CLASS 1 - Residential and commercial wall and bare foot traffic.
CLASS 2 - Wall and residential bath floor, and soft soled traffic.
CLASS 3 - All residential floors and light commercial floors.
CLASS 4 - Medium commercial, light industrial and institutional, moderate soiling.

CLASS 5 - Extra heavy traffic, abrasive dirt, chemically more resistant.

Water Absorption ASTM C-373 rates the amount of water absorption of bisque. 7-20% Non-vitreous, 3-7% Semi- vitreous, 0.5-3% Vitreous, less than .5% Porcelain Impervious. Only tiles with less than .5% water absorption can be used on shower floors, spas, and outdoor areas.

Coefficient of Friction (COF) The coefficient of friction rates the slip resistance of a tile. ADA recommends a test rating of .6 (wet or dry) on floors and .8 for ramps. Breaking Strength – Chip Resistance The breaking strength test is a good indicator of the chip resistance of a tile because it tests the strength and density of the bisque. Using applicable ASTM test, ceramic floor tile typically has a breaking strength of 250 lbs. This can be less for poor quality tiles, higher with porcelain tiles (up to 400lbs).